# Onboard Computer Options

## Guides

http://shervinemami.info/embeddedVision.html

| Board | Raspberry Pi v3 Model B | ODROID-XU4 | Up Squared (oem) | UDOO x86 Advanced PLUS (oem) | Jetson TX1/2 + CTI Orbitty or Auvidia J120 | DJI Manifold (Tegra K1) |
| --- | --- | --- | --- | --- | --- | --- |
| Release | 2016 | Feb 2016 | May 2017 | Q2 2017 | K1: Q2 2014 X1: Q2 2015 X2: Q1 2017 | K1: Q2 2014 |
| CPU | Broadcom BCM2387 chipset, 1.2GHz 64 bit Quad-Core ARM Cortex-A53.802.11 b/g/n | Samsung Exynos5422 Cortex™-A15 2Ghz and Cortex™-A7 Octa core CPUs | Various intel | Braswell series Quad-core Intel Celeron N3160, up to 2.24GHz (INTEL PENTIUM N3710 2.56 GHZ on Ultra) Intel Quark SER core 32MHz microcontroller | Tegra X1 ( 4 CPU-cores, 64-bit ARM® CPU 4x A57 2MB L2 ARMv8 ARM Cortex-A57 quad-core + ARM Cortex-A53 quad-core (64-bit) A57: 1.9 GHz A53: 1.3 GHz | NVIDIA 4-Plus-1™ Quad-Core ARM Cortex-A15 "r3" 2.3 GHz |
| Architecture | ARMv8 64bit | ARMv8 64bit | x86-64 | x86-64 | ARMv8 64bit | ARM 32bit |
| Memory | 1GB LPDDR2 | 2GB LPDDR3 | 8GB LPDDR4 | 4 GB DDR3L Dual Channel (8GB on Ultra) | 4 GB 64 bit LPDDR4 25.6 GB/s | 8GB 64bit DDR3L and LPDDR3 17 GB/s ARM Large Physical Page Extension (LPAE) supports 1 TiB (240 bytes). The 8 GiB limitation is part-specific. |
| GPU | Dual Core VideoCore IV® Multimedia Co-Processor. Provides Open GL ES 2.0, hardware-accelerated OpenVG, and 1080p30 H.264 high-profile decode. Capable of 1Gpixel/s, 1.5Gtexel/s or 24GFLOPs with texture filtering and DMA infrastructure | Mali-T628 MP6 Hardkernel: OpenGL ES 3.1 OpenCL 1.2 Jiffy: OpenGLES 3.0 OpenCL 1.1 | Intel Gen 9 HD 500 or HD 505 | Intel HD Graphics 400, UP TO 640 MHZ, (INTEL HD GRAPHICS 405 UP TO 700 MHZ on Ultra) | NVIDIA Maxwell 256-core GPU DX-12, OpenGL 4.5, NVIDIA CUDA®, OpenGL ES 3.1, AEP, and Vulkan | NVIDIA® Kepler™ Architecture 192 NVIDIA CUDA® Cores |
| OS | Raspbian, linux, Windows 10 IOT | Linux Kernel 4.9 LTS Android <= 7.1 Nougat | Win IOT, Win 10, Ubuntu, Ubilinux, Android Mrshmllw | All x86_64 OS | Ubuntu | LX4T (Ubuntu variant) |
| POWER | 5V, 2.5A | 5V, 4A |  | 12V | 9-15V TX1: 6.5-15W Orbitty: 2-6W K1: 20 nm SOC - TSMC Isolated Power Rails, Fourth-Generation Cluster Switching |  |
| TDP | 12.5 W max | 20 W max |  | 5 or 6 Watts | TDP 15 watts, with average power consumption less than 10 watts | Power consumption: 8 watts |
| IO | 802.11n Wifi Bluetooth 4.1 Bluetooth LE 4xUSB2.0 40pin GPIO HDMI 1xGbE 3.5mm audio + composite video CSI(Camera) DSI(Display) | eMMC5.0 socket HDMI MicroSD Ethernet 4pin Serial 30pin GPIO 12pin GPIO 2x USB3.0 1x USB2.0 Note: GPIO pins at 1.8V, needs shifter shield to raise to 3.3V or 5v | SATA3 2xHDMI Displayport 2xGbE 1x2laneCSI 1x4laneCSI Pi 40pim GPIO | Ethernet HDMI 2x UART 3x USB3.0 eMMC storage MicroSD SATA Arduino 20pin IR LPC 2xI2C RTC Battery 3.5 mm jack M.2 Key E slot for optional Wireless modules | Orbitty: 1xGbE 1xUSB3.0 1xUSB2.0 HDMI 1xMicroSD 2xUART I2C 4xGPIO J120-IMU: HDMI GbEthernet 2xUSB3.0 1xUSB2.0 1xCSI-2 for B102 2x UART 2x DSI SPI/GPIO MicroSD I2C MPU9250-IMU included | CAM_IN and CAM_OUT HDMI - 4K (UltraHD, 4096x2160) |
| VIDEO IO |  | HDMI 1080p via HDMI cable(H.264+AAC based MP4 container format) |  | HDMI 2xminiDP++ HW Video decode: H.265/HEVC, H264, MPEG2, MVC, VC-1, WMV9, JPEG, VP8; HW Video encode: H.264, MVC, JPEG | TX1: H.265, VP9 4K 60 fps Video 4k H.265, 4k VP9, 4k H.264 4K x 2K @60 Hz, 1080p @120 Hz HDMI 2.0 60 fps, HDCP 2.2 |  |
| Size | 85mm x 56mm H: 17mm | 83mm x 58mm H: 22mm | 85.6mm x 90mm | 120mm x 85mm | TX1 + Orbitty/J120: 87mm x 50mm Height: full stack: 30.18mm | 23x23 FCBGA 16x16 S-FCCSP 15x15 FC PoP |
| PRICE | $64.35 AUD jiffy(au) | $149.95 AUD jiffy(au) | $145 USD to $269 USD | $253.54 AUD mouser(au) | TX1: $759 AUD With DK board ubuy(us,au) TX2: $790 AUD with DK board newegg(au) xenon(au) Orbitty: $356 USD $474.54 AUD ($230.61 AUD + $243.93 AUD shipping, flat rate) wdl(us) Orbitty+TX1: $823.00 USD $1097.04 AUD wdl(us) J120: € 268,90 $379.64 AUD ($309.19 AUD + $70.45 AUD shipping, flat rate) | $859 AUD risabove(au) |
| USE Cases | Delivery | HD Image capture + image processing or advanced navigation Lacks CSI Can get USB images |  | Fast + Parallel CPU processing | Multi Image Processing | Single Image Processing |

# Combinations

## Earl-Brain

https://erlerobotics.com/blog/product/erle-brain-3/

BeagleBone Black or Raspberry Pi Zero + PixHawk Fire Cape

Simple linux autopilot

http://ardupilot.org/dev/docs/beaglepilot.html

# Links and Notes

## Raspberry Pi

## ODROID-U3

http://hardkernel.com/main/products/prdt_info.php?g_code=G138745696275

## ODRIOD-XU4

http://hardkernel.com/main/products/prdt_info.php?g_code=G138745696275

https://jiffyshop.com.au/SBC/21-odroid-xu4

## Gumstix Overo

## Udoo x86

http://www.udoo.org/udoo-x86/

http://au.mouser.com/new/udoo/udoo-x86/

Pre-Order from main distributor

## NVIDIA Jetson TK1

http://www.fishpond.com.au/Electronics/NVIDIA-Jetson-TK1-Development-Kit/0850438005456

# NVidia Jetson TX1

http://www.nvidia.com/object/tegra-x1-processor.html

http://www.nvidia.com/object/embedded-systems-dev-kits-modules.html

http://diydrones.com/profiles/blogs/nvidia-jetson-tx1-carrier-boards-overview

http://ardupilot.org/dev/docs/companion-computer-nvidia-tx1.html

$759 AUD With development kit board

https://www.u-buy.com.au/catalog/product/view/id/1046146/s/nvidia-jetson-tx1-development-kit-proprietary-ddr4-motherboards-945-82371-0000-000?sku=B017NWO6LG&store=US

## NVidia Jetson TX2

$790 AUD with Developer Kit board

https://www.newegg.com/global/au/Product/Product.aspx?Item=N82E16813190007&Tpk=32gb%20ram

## AuVidia J120

https://auvidea.com/purchase/

## Orbitty Carrier with Jetson TX1

http://connecttech.com/product/orbitty-carrier-for-nvidia-jetson-tx2-tx1/

http://www.wdlsystems.com/Computer-on-Module/Carrier-Boards/CTI-Orbitty-Carrier-for-NVIDIA-Jetson-TX1.html

http://www.connecttech.com/pdf/CTIM-ASG003_Manual.pdf

http://www.nvidia.com/object/embedded-systems-dev-kits-modules.html

## DJI Manifold

http://www.nvidia.com/object/tegra-k1-processor.html

$859 AUD

http://www.riseabove.com.au/dji-manifold

http://airsoc.com/articles/view/id/573f5ecc313944b4038b4567/nvidia-jetson-tx1-carrier-boards-overview